XYZ Company — Bonalbo Branch

Small Office Network Design

Project Summary — What I Built & How I Did It

Tool: Cisco Packet Tracer   |   Routing: Router-on-a-Stick   |   IP: VLSM /26

What Is This Project?

This is a Small Office / Home Office (SOHO) network design project built in Cisco Packet Tracer for XYZ Company's new Bonalbo branch. The branch needed a network that is completely separate from the company's headquarters.

The goal was to connect three departments — Admin/IT, Finance/HR, and Customer Service/Reception — so they can communicate with each other while staying logically separated using VLANs, with wireless access for all users and automatic IP assignment via DHCP.

Requirements I Had to Meet

- Use only one Cisco router and one Cisco switch

- Create 3 separate departments, each in its own VLAN

- Provide wireless access in every department

- All devices must get their IP addresses automatically (DHCP)

- All departments must be able to communicate with each other

- Base network given by ISP: 192.168.1.0

Devices I Used

| Device | Model | Quantity | Role |
| --- | --- | --- | --- |
| Router | Cisco 2911 (R1) | 1 | Routing between VLANs + DHCP server |
| Switch | Cisco 2960-24TT (SW1) | 1 | Switching + VLAN separation |
| Access Points | AccessPoint-PT | 3 | One per department for WiFi |
| PCs | PC-PT | 3 | One wired PC per department |
| Laptops | Laptop-PT | 2–3 per dept | Wireless clients |
| Smartphones | Smartphone-PT | 3 | Wireless mobile devices |
| Printers | Printer-PT | 3 | One per department |

How I Divided the IP Addresses

I used VLSM (Variable Length Subnet Masking) to split the 192.168.1.0 base network into three equal /26 subnets — one per department. A /26 gives 62 usable host addresses per department, which is more than enough for a small office.

| VLAN | Dept | Subnet | Mask | Gateway | Host Range | Broadcast | Hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | Admin/IT | 192.168.1.0/26 | 255.255.255.192 | 192.168.1.1 | 192.168.1.2–.62 | 192.168.1.63 | 62 |
| 20 | Finance/HR | 192.168.1.64/26 | 255.255.255.192 | 192.168.1.65 | 192.168.1.66–.126 | 192.168.1.127 | 62 |
| 30 | CS/Reception | 192.168.1.128/26 | 255.255.255.192 | 192.168.1.129 | 192.168.1.130–.190 | 192.168.1.191 | 62 |

What I Implemented — Step by Step

1.  Basic Device Security (Both Router & Switch)

The first thing I configured on both devices was security settings so the devices are protected:

- Set hostname to R1 and SW1

- Set enable secret password — protects privileged mode

- Created a local username and password for login

- Enabled service password-encryption — hides all passwords in config

- Disabled IP domain-lookup — stops the router freezing when you mistype a command

- Set a banner MOTD warning message — displayed before anyone logs in

- Set console and VTY lines to require login

2.  VLAN Creation on the Switch

I created 3 VLANs on SW1 to logically separate each department, plus a native VLAN 1000 for the trunk link:

- VLAN 10: ADMIN-IT — for Admin/IT department devices

- VLAN 20: FINANCE-HR — for Finance/HR department devices

- VLAN 30: CS-RECEPTION — for Customer Service department devices

- VLAN 1000: NATIVE — used as native VLAN on trunk (security best practice)

3.  Switch Port Assignment

I assigned each physical switch port to its correct VLAN in access mode:

- Fa0/1–3: Access ports assigned to VLAN 10 (Admin/IT)

- Fa0/4–6: Access ports assigned to VLAN 20 (Finance/HR)

- Fa0/7–9: Access ports assigned to VLAN 30 (CS/Reception)

- Gig0/1: Trunk port to router — carries all VLANs tagged

4.  Trunk Port (Switch → Router)

The uplink from SW1 Gig0/1 to R1 Gig0/0 is configured as a trunk so all VLAN traffic can travel on one cable:

- Mode set to trunk with 802.1Q encapsulation

- Native VLAN set to 1000 (not default VLAN 1 — avoids VLAN hopping attacks)

- Only VLANs 10, 20, 30, 1000 allowed on trunk — unused VLANs pruned

5.  Router-on-a-Stick (Inter-VLAN Routing)

Since I only had one router and one switch, I used Router-on-a-Stick. The single physical link carries all VLAN traffic and the router uses sub-interfaces — one per VLAN — to route between them:

- Gig0/0.10: encapsulation dot1Q 10 — gateway 192.168.1.1 for Admin/IT

- Gig0/0.20: encapsulation dot1Q 20 — gateway 192.168.1.65 for Finance/HR

- Gig0/0.30: encapsulation dot1Q 30 — gateway 192.168.1.129 for CS/Reception

Each sub-interface acts as the default gateway for its department. When a PC in Admin/IT wants to reach Finance/HR, the packet goes up to the router, gets routed, and comes back down to the correct VLAN.

6.  DHCP — Automatic IP for All Devices

I configured DHCP on the router so every device gets an IP address automatically. I created one pool per VLAN and excluded the first few addresses in each pool for infrastructure devices (gateway, AP, printer):

- Excluded: 192.168.1.1–.3 (VLAN 10), .65–.67 (VLAN 20), .129–.131 (VLAN 30)

- Pool ADMIN-IT: network 192.168.1.0/26, gateway 192.168.1.1

- Pool FINANCE-HR: network 192.168.1.64/26, gateway 192.168.1.65

- Pool CS-RECEPTION: network 192.168.1.128/26, gateway 192.168.1.129

- DNS server set to 8.8.8.8 (Google) on all pools

7.  Wireless — One Access Point Per Department

I added one Access Point per department, each connected to the switch on an access port in the correct VLAN. The APs provide WiFi for laptops, smartphones, and other wireless devices:

| AP | Department | SSID | Switch Port | VLAN |
| --- | --- | --- | --- | --- |
| AP1 | Admin / IT | XYZ-AdminIT | Fa0/3 | 10 |
| AP2 | Finance / HR | XYZ-FinanceHR | Fa0/6 | 20 |
| AP3 | CS / Reception | XYZ-CSReception | Fa0/9 | 30 |

- WPA2-PSK security on all SSIDs — wireless traffic is encrypted

- Wireless devices connect to their department AP and get an IP from the same DHCP pool as wired devices in that VLAN

How I Verified It Works

After configuring everything, I ran these checks to confirm the network is working:

- show ip int brief (R1): Confirmed all 3 sub-interfaces are up/up with correct IPs

- show vlan brief (SW1): Confirmed VLANs 10, 20, 30 are active with correct ports assigned

- show interfaces trunk (SW1): Confirmed Gig0/1 is trunking with VLANs 10, 20, 30, 1000

- show interfaces status (SW1): Confirmed Fa0/1–9 are connected to the right VLANs

- ipconfig on PCs: Confirmed every device received a DHCP address in the correct subnet

- Ping PC1 → PC2: Inter-VLAN routing working — Admin/IT can reach Finance/HR

- Ping PC1 → PC3: Inter-VLAN routing working — Admin/IT can reach CS/Reception

- Ping PC2 → PC3: Inter-VLAN routing working — Finance/HR can reach CS/Reception

- Wireless ping: Smartphones and laptops can ping across departments via WiFi

Challenges & What I Learned

- Router-on-a-Stick requires the physical interface (Gig0/0) to be up with "no shutdown" — forgetting this means no sub-interface works even if configured correctly

- The native VLAN on the switch trunk and the router's encapsulation dot1Q native VLAN must match — mismatch causes CDP warnings and traffic issues

- DHCP excluded addresses must be configured before the pools, otherwise the router may lease out the gateway IP to a host

- Each AP sits on an access port (not trunk) in Packet Tracer — the AP internally handles wireless-to-wired bridging for its VLAN

- Changing native VLAN from 1 to 1000 is a security best practice that prevents VLAN hopping attacks

Quick Summary

| What | Details |
| --- | --- |
| Network type | Small Office / Home Office (SOHO) |
| Routing method | Router-on-a-Stick — 802.1Q sub-interfaces on R1 |
| Number of VLANs | 3 (VLAN 10, 20, 30) + VLAN 1000 native |
| IP scheme | 192.168.1.0 → three /26 subnets (VLSM) |
| DHCP | Centralised on router — one pool per VLAN |
| Wireless | 3 APs — one per department — WPA2-PSK |
| Security | Enable secret, local login, encrypted passwords, MOTD, native VLAN 1000 |
| All depts communicate? | Yes — verified with ping tests across all 3 VLANs |
| Auto IP on all devices? | Yes — DHCP leases confirmed on all PCs, laptops, smartphones |

Built with Cisco Packet Tracer as part of a networking fundamentals project. All requirements successfully implemented and tested.